# Ultimate Spice Rack Build Guide

3D Printing & Assembly (Plates A-C)

This guide includes complete steps for printing, assembling, and testing your Ultimate Spice Rack build. Each section provides checkboxes for tracking progress and placeholders for photos of each stage.

## 1) Bill of Materials

[ ] Part: #224 Buna-N O-Ring (70A durometer)
    - Spec: 1-3/4" ID, 2" OD, 1/8" width
    - Qty: 2 (spares optional)

[ ] Lubricant: Lucas Oil White Lithium Grease (Tube Only)
WARNING: Do not use aerosol/spray grease; propellants can crack 3D printed plastic.

[ Photo Placeholder: Hardware Layout ]

## 2) Printing Guide (3 Plates)

Plate A - The Base (Enclosure)
[ ] PART_TO_PRINT = 1
[ ] TRACTION_MODE = 1
[ ] Material: PLA or PETG
[ ] 3 Walls, 15-20% Infill, 5mm Brim, No Supports
[ ] Let bed cool fully before removing

[ Photo Placeholder: Base Print ]

Plate B - The Gears (Tires)
[ ] PART_TO_PRINT = 2
[ ] TRACTION_MODE = 1 (Critical for #224 O-rings)
[ ] Material: PLA or PETG
[ ] 3 Walls, 20% Infill, Skirt Adhesion, No Supports

[ Photo Placeholder: Gears Print ]

Plate C - The Handle (Lozenge)
[ ] PART_TO_PRINT = 3
[ ] Orientation: Flat top down
[ ] Brim adhesion, No Supports

[ Photo Placeholder: Handle Print ]

## 3) Assembly Instructions

Step 1 - Prep the Gears
[ ] Inspect grooves on large gears
[ ] Clean out strings/blobs
[ ] Install O-rings and perform roll test

[ Photo Placeholder: Gear Assembly ]

Step 2 - Prep the Base
[ ] Clean and sand pins
[ ] Apply thin layer of grease
[ ] Check wobble; add rubber feet if needed

[ Photo Placeholder: Base Prep ]

Step 3 - Final Assembly
[ ] Drop in gears (idler + large gears)
[ ] Spin test for smooth motion
[ ] Attach handle (tight fit)

[ Photo Placeholder: Full Assembly ]

## 4) The Spice Test

[ ] Place spice bottle on gear (rubber O-ring contact)
[ ] Turn handle - check smooth rotation
[x] Rubber should grip bottle, not slip

[ Photo Placeholder: Spice Test in Action ]